- List 5 common BIOS or POST error messages you might see when starting a PC and write what hardware or configuration issue each one usually points to.

ANS:

| No. | BIOS/POST Error Message | Usually Indicates |
| --- | --- | --- |
| 1 | CMOS Checksum Error | CMOS battery is weak/dead or BIOS settings are corrupted. |
| 2 | Keyboard Error / Keyboard Not Detected | Keyboard is disconnected, faulty, or USB/PS/2 port has an issue. |
| 3 | Memory Error / RAM Not Detected | RAM is loose, incompatible, faulty, or not installed correctly. |
| 4 | CPU Fan Error | CPU fan is disconnected, not spinning, or has a cooling problem. |
| 5 | No Boot Device Found | HDD/SSD is not detected, boot order is incorrect, or the operating system/boot files are missing. |

- Given this scenario: On boot, a PC beeps 3 times in a repeating pattern and does not display anything on the screen. Research and write what this POST beep code typically means for an AMI BIOS, and suggest two troubleshooting steps.

ANS:

POST Beep Code: 3 Beeps (AMI BIOS)

- Meaning: In an AMI BIOS, 3 short beeps typically indicate a RAM (memory) problem, such as faulty or improperly seated RAM.

  - Power off the PC, remove the RAM module, clean the contacts gently, and reseat it firmly.

  - Test the RAM one module at a time or try another known-good RAM module to identify a faulty stick.

  - Access the BIOS setup utility on any available PC or virtual machine, take a screenshot of the main BIOS screen, and identify the motherboard manufacturer and BIOS version.

ANS: 1.  Restart the PC.

2. During startup, repeatedly press F2, Delete, F10, or Esc depending on the system.

3.  Open the BIOS/UEFI Setup Utility.

4.  On the Main/System Information screen.

4. You are given a screenshot of a POST error message that says 'CMOS checksum error – Defaults loaded.' Explain what this means and describe how you would resolve this issue in a real system.<br><br><em><strong>Hint:</strong> Think about the CMOS battery and BIOS settings reset.</em>

ANS:

Meaning:
“CMOS checksum error – Defaults loaded” means the BIOS detected that the saved CMOS/BIOS settings are corrupted, invalid, or have been lost. As a result, the BIOS loads its default settings.

How to resolve it:

- Enter the BIOS/UEFI Setup and check the date, time, boot order, and other required settings. Save the correct settings and restart.

- If the error appears again, replace the CMOS battery (usually a CR2032), then enter BIOS again, configure the settings, and save them.