- Draw a labeled diagram of a Cat6 Ethernet cable, showing the color order of wires in the T568B standard and labeling the RJ-45 connector pins.

ANS:

RJ-45 Connector (Pins 1–8)

┌─────────────────────────────┐

Pin 1   │ White-Orange                │

Pin 2   │ Orange                      │

Pin 3   │ White-Green                 │

Pin 4   │ Blue                        │

Pin 5   │ White-Blue                  │

Pin 6   │ Green                       │

Pin 7   │ White-Brown                 │

Pin 8   │ Brown                       │

└─────────────────────────────┘

T568B Color Order:

1 → White-Orange

2 → Orange

3 → White-Green

4 → Blue

5 → White-Blue

6 → Green

7 → White-Brown

8 → Brown

- List three types of network cables (such as Cat5e, Cat6, and fiber optic) and for each, mention one real-world app or device (like a WiFi router, Zomato POS terminal, or IPL streaming server) where it would be used, explaining why.

ANS:

| Cable Type | Real-World Use | Why Used |
| --- | --- | --- |
| Cat5e | Wi-Fi Router | Low-cost and suitable for normal network connections. |
| Cat6 | Zomato POS Terminal | Provides faster and reliable wired communication. |
| Fiber Optic | IPL Streaming Server | Provides very high speed and supports long-distance data transmission. |

- Watch a YouTube video showing how to crimp an RJ-45 connector onto a network cable, then write down the step-by-step process in your own words and mention two common mistakes to avoid.

ANS:

- Cut the Ethernet cable to the required length and remove the outer jacket.

- Untwist and straighten the eight wires.

- Arrange the wires according to the T568B color order: White-Orange, Orange, White-Green, Blue, White-Blue, Green, White-Brown, Brown.

- Cut the wires evenly and insert them fully into the RJ-45 connector.

- Make sure the cable jacket enters the connector for proper support.

- Place the connector into the crimping tool and press it firmly.

- Repeat the process on the other end and test the cable with a LAN cable tester.

Two Common Mistakes to Avoid:

Using the wrong wire color order.

Not inserting the wires fully or making a loose crimp.

- Given the following scenario: You are setting up a gaming zone for an IPL screening event and need to connect multiple screens and consoles. Choose the appropriate cable and connector types for high-speed, reliable connections, and explain your choices.

ANS:

For an IPL screening gaming zone HDMI cable with HDMI connectors, is suitable for connecting gaming consoles to TVs or monitors. It provides high-quality video and audio with fast data transmission.

For network connections, Cat6 Ethernet cable with RJ45 connectors is suitable. It provides a fast, stable and reliable network connection between consoles, computers, switches and routers.

- Find and share a photo(from your home,college,or online) of any network cable or connector in use (such as behind a WIFI router or at a cafes POS counter),and describe what type it is and what devices it connects.<br><br><em><strong>hint:</strong>look for blue or yellow cables with clear plastic ends (RJ-45) or thin round cables for fiber optics.</em>

ANS:

I found a blue Cat6 Ethernet cable with an RJ-45 connector connected to a Wi-Fi router.

- Cable Type: Cat6 Ethernet Cable

- Connector Type: RJ-45

- Connected Devices: Wi-Fi Router and Computer