- List 3 key differences between Ethernet cables and fiber optic cables, and give one real-world example where each type would be used (for example, home WiFi router, data center, college campus backbone).

ANS:

| No. | Ethernet Cable | Fiber Optic Cable |
| --- | --- | --- |
| 1. | Uses copper wires to transmit data. | Uses light signals through glass/plastic fiber. |
| 2. | Usually suitable for shorter distances and is generally cheaper. | Suitable for longer distances and is faster over long links. |
| 3. | More affected by electromagnetic interference (EMI). | Not affected by electromagnetic interference. |

- Draw and label a diagram showing the internal wiring of a straight-through Ethernet cable (use T568B wiring standard), indicating the color of each wire and its pin number.

ANS:

- Watch a YouTube video demonstrating how to use an RJ45 crimper, then write a step-by-step guide (in your own words) on how to attach an RJ45 connector to a CAT6 cable.

ANS:

- Cut the CAT6 cable to the required length.

- Remove about 2–3 cm of the outer cable jacket carefully.

- Separate and straighten the eight wires.

- Arrange the wires according to the T568B standard.

- The color order is: White/Orange, Orange, White/Green, Blue, White/Blue, Green, White/Brown, Brown.

- Cut the wire ends evenly so they are the same length.

- Insert all eight wires into the RJ45 connector in the correct order.

- Check that all wires reach the end of the connector and remain in the correct order.

- Insert the RJ45 connector into the crimping tool and press firmly.

- Finally, test the cable using a LAN cable tester to confirm that the connection is working correctly.

- Imagine you are setting up a gaming café with 10 PCs for LAN gaming. Decide which cable type (Ethernet or fiber optics) and which tool (RJ45 crimper or fiber cleaver) you would use, and explain your choices in 3-4 sentences.<br><br><em><strong>Hint:</strong> Consider speed, distance, and budget for your answer.</em>

ANS:

For a gaming cafe with 10 PCs, I would choose CAT6 Ethernet cables because they provide sufficient speed for LAN gaming and are affordable. I would use an RJ45 crimper to attach the RJ45 connectors to the cables. Since the PCs are usually located within a short distance of the network switch, fiber optic cables are not necessary. This setup is cost-effective, reliable, and easy to install.