- List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

ANS:

| Hardware Issue | Typical Symptom |
| --- | --- |
| 1. No display on monitor | Blank screen, no POST beep, monitor shows "No Signal." |
| 2. Faulty RAM | PC beeps repeatedly during startup, random crashes, or fails to boot. |
| 3. Hard drive/SSD failure | Operating system does not load, "Boot Device Not Found" error, unusual clicking noises (HDD). |
| 4. Overheating CPU | PC shuts down unexpectedly, freezes, restarts, or runs very slowly with loud fan noise. |
| 5. Power supply (PSU) failure | PC does not turn on, no lights or fan activity, or powers off suddenly during use. |

- Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

ANS:

- Check the power outlet

- Inspect the charger and cable

- Check the charging port

- Try another compatible charger

- Perform a power reset

- Check the battery physically

- Use a multimeter to test the charger

- Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

ANS:

- Set the multimeter to DC Voltage (V⎓).

- Connect the black probe to GND (negative) and the red probe to +5V (positive) of the USB output.

- Turn on the USB charger and observe the multimeter reading.

- Measured voltage: 5.02 V DC

- Expected range: 4.75 V to 5.25 V

- Result: 5.02 V is within the expected range, so the charger output voltage is normal.

- A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

ANS:

1.  Shut down the PC and unplug the power supply from AC power.

2.  Disconnect the ATX power connector from the motherboard.

3.  Set the multimeter to DC Voltage (V⎓).

4.  Identify the yellow wire (+12V) and black wire (GND).

5.  With the PSU powered on using an appropriate safe test setup, place the red   probe on the yellow wire and the black probe on the black wire.

6.  Record the voltage shown on the multimeter.