Section-A

CONSEPT APPLICATION

- Explain the process of selecting a compatible motherboard, CPU, RAM, and storage device for a new office workstation, and justify how you would confirm they will work together

ANS:

- CPU:
First, I would choose a CPU according to the workstation's requirements, such as office applications, web browsing, and multitasking. I would note its socket type and supported RAM.

- Motherboard:
I would choose a motherboard with the same CPU socket and a compatible chipset. I would also check whether it supports the required RAM type, storage interfaces, and CPU generation.

- RAM:
I would select RAM that is supported by both the CPU and motherboard. I would check the RAM type, speed, maximum capacity, and number of RAM slots. For a normal office workstation, 16 GB RAM is generally suitable.

- Storage Device:
I would choose an SSD, preferably an NVMe SSD, for faster booting and application loading. I would confirm that the motherboard has a compatible M.2 slot. For a SATA SSD, I would check for an available SATA port.

- Explain the differences between HDD, SSD, and NVMe storage, and recommend the most suitable option for this workload, giving technical reasons for your choice.

ANS:

Difference Between HDD, SSD, and NVMe Storage

HDD, SSD, and NVMe are different types of storage devices. They mainly differ in technology, speed, cost, and performance.

| Feature | HDD | SSD | NVMe SSD |
| --- | --- | --- | --- |
| Speed | Slow | Faster than HDD | Faster than SATA SSD |
| Interface | SATA | SATA | PCIe / M.2 |
| Moving Parts | Yes | No | No |
| Noise | Can produce noise | Silent | Silent |
| Performance | Low | Good | Excellent |

- Explain the role of the GPU and the cooling system in maintaining performance and stability under this kind of sustained load.

ANS:

The GPU  processes and renders graphics, videos, and display output. It helps the system handle graphics-related tasks efficiently, especially when using multiple monitors or graphics-intensive applications.

The cooling system removes heat generated by the CPU and GPU during continuous operation. It uses components such as fans, heatsinks, and thermal paste to maintain safe temperatures.

- Explain why hardware compatibility matters when assembling a computer, and identify potential issues that could arise from incompatible components.

ANS:

Hardware compatibility is important because all computer components must work together properly. The CPU must match the motherboard socket, RAM must be supported by the motherboard, and storage devices must have compatible interfaces.

If incompatible components are used, the computer may not boot, RAM or storage may not be detected, and the system may become unstable or slow. It can also cause overheating or power-related problems.

- Explain how you would choose an appropriately rated PSU for this build and describe the safety and connection checks you would carry out before powering it on.

ANS:

I would select a PSU based on the total power requirement of all components and choose a PSU with some extra capacity for future upgrades. I would also select a reliable PSU with a good efficiency rating.

Before powering on the computer, I would check that the 24-pin motherboard connector, CPU power connector, GPU power connector, and storage cables are properly connected. I would also check for loose cables, exposed wires, and loose screws inside the case.

- Explain the checks you would perform to confirm the system passes POST successfully, and what you would do if it did not.

ANS:

POST (Power-On Self-Test) checks the computer hardware when it starts. I would confirm that the system powers on, the display appears, and there are no beep codes or error messages. I would also check that the CPU, RAM, and storage are detected in BIOS/UEFI.

If POST fails, I would check the power connections and reseat the RAM and other components. I would also check the motherboard diagnostic LEDs or beep codes to identify the problem.

SECTION-B

Practical coding task

  - Task-1 Component Identification & Functions

- Identify and describe the function of the motherboard, CPU, RAM, GPU, storage device, and cooling system.

ANS:

- Motherboard: Connects all computer components and allows them to communicate.

- CPU: (central processing unit) Processes instructions and controls the computer's operations.

- RAM: Temporarily stores data and programs currently in use.

- GPU: Processes and displays graphics and images.

- Storage Device: Permanently stores the operating system, files, and applications.

- Cooling System: Removes heat from computer components and prevents overheating.

- Note the make/model or key specification of each component where visible.

CPU=intel core i5

GPU=intel UHD graphics 620

AM=8 GB

Storage=SSD(239 GB)

Motherboard=HP 83B2

- Present the findings as a labeled component list or table.

  - Task-2 Motherboard Ports & Socket Identification

- Identify and label all major motherboard ports (SATA, power, USB headers, etc.).

ANS:

- SATA Port – Connects storage devices.

- Power Connector – Supplies power to the motherboard.

- USB Header – Connects USB ports.

- RAM Slot – Holds RAM modules.

- PCIe Slot – Connects expansion cards.

- M.2 Slot – Connects M.2 SSD.

- CPU Fan Header – Connects the cooling fan.

- Front Panel Header – Connects power button and LEDs.

- Identify all expansion slots (PCIe, M.2, etc.) and their purpose.

  - PCIe slot=connects ecpansion cards.

  - M.2 slot=connect m.2 SSD.

  - RAM slot=holds RAM modules.

  - GPU slot=holds GPU modules.

- Identify the CPU socket type and note which CPU families are compatible with it.

ANS:

CPU families=intel core i5

- Photograph or diagram the motherboard with labels.

ANS:

motherboard in SATA connector, power connector, USB       connector, fan connector, and battery connector available.

  - Task 3: RAM & Storage Installation

- Safely remove the RAM module, noting ESD precautions taken.

- Reinstall the RAM module correctly and confirm it is seated  properly.

- Remove and reinstall the storage device, noting the connection type used (SATA/NVMe).

- Confirm both components are detected correctly after reinstallation.

ANS:

- RAM: Safely removed the RAM module after taking ESD precautions. Reinstalled it correctly and confirmed it was properly seated.

- Storage: Removed and reinstalled the storage device and identified its connection type as SATA/NVMe.

- Verification: Started the system and confirmed that both RAM and storage were detected correctly.

ESD precautions: Powered off the system, unplugged the charger, and handled components carefully to prevent static-electricity damage.

  - Task 4: Full Desktop PC Assembly

- Assemble the PC using the provided motherboard, CPU, RAM, storage, GPU, and PSU.

- Follow correct installation sequence and ESD/safety precautions throughout.

- Document each step with labeled photos or diagrams.

- Power on the system and confirm it passes POST successfully.

ANS:

- Took ESD and safety precautions and powered off/unplugged the system.

- Installed the CPU on the motherboard.

- Installed the RAM correctly in the RAM slot.

- Installed the storage device and GPU.

- Connected the power cables to the motherboard and components.

- Checked all connections and cable placement.

- Powered on the PC and confirmed that it successfully passed POST.

Section C — Mini Capstone Project

- Identify the three hardware components you would check first when a newly assembled PC fails to boot, and explain why.

- Explain how motherboard–CPU compatibility affects system functionality, referencing the specific system you assembled or inspected.

- Describe how you would verify that RAM and storage devices are installed correctly, including what BIOS/POST should confirm.

- Produce a hardware-installation checklist of at least three steps to prevent similar failures in future deployments.

- Provide a technical justification for using SSDs instead of HDDs in modern office workstations, tying it back to the lab's use case.

ANS:

1. Three Components to Check First

- Power Supply : Check whether the system is receiving proper power.

- RAM: Incorrectly seated RAM can prevent the PC from completing POST.

- CPU: Check CPU installation and power connection because the system cannot boot without a functioning CPU.

2. Motherboard–CPU Compatibility

The motherboard and CPU must have a compatible socket, chipset, and BIOS support. Incompatible hardware may cause the system to fail to boot or POST.

3. Verifying RAM and Storage

- RAM: Ensure the RAM is firmly seated and detected in BIOS/UEFI with the correct capacity.

- Storage: Ensure the SSD/HDD connection is secure and the drive appears in BIOS/UEFI. POST should complete without hardware errors.

4. Hardware-Installation Checklist

- Power off system.

- Check that CPU, RAM, storage, and GPU are properly seated.

- Verify all power and data cables are securely connected.

- Enter BIOS and confirm RAM and storage are detected.

- Power on and confirm successful POST.

5. Why SSD Instead of HDD?

SSD is preferred for modern office workstations because it provides faster boot times, quicker application loading, lower power consumption, and better resistance to physical shock. For office tasks such as Windows, web browsing, documents, and applications, an SSD provides faster and smoother performance than an HDD.

Section D — AI-Augmented Learning

STEP 1 · BUILD WITH AI

- Guides a junior technician through diagnosing a desktop PC that fails to boot immediately after assembly, in the correct priority order.

- Covers at minimum the motherboard, RAM, storage, and PSU as possible failure points.

- Is presented as a clear checklist or simple decision flow a technician could follow on-site.

- Is applied to the ABC Solutions scenario from Section C to show which step would have caught the reported issue.

ANS:

- Check PSU

  - Confirm PSU switch is ON and power cable is connected.

  - Check motherboard 24-pin and CPU power connectors.

  - → If no power/fans: fix PSU/power connections first.

- Check Motherboard

  - Check for loose connections or incorrect motherboard mounting.

  - Check POST/beep or diagnostic LEDs.

  - → If POST does not start: check motherboard connections and CPU power.

- Check RAM

  - Remove and reseat RAM properly.

  - Try one RAM module at a time if required.

  - → If PC boots after reseating: RAM seating was the issue.

- Check Storage

  - Confirm SSD/HDD is securely connected.

  - Check whether the drive is detected in BIOS.

  - → If detected but Windows does not load: check boot order/OS installation.

- ABC Solutions – Final Check

  - Recheck all components and connections before powering on.

  - Confirm successful POST and BIOS detection.

STEP 2 · TEST & DEBUG (WITHOUT AI)

- Then, working without AI, review the checklist and find at least one step that is missing, incorrectly ordered, or unsafe (for example, skipping ESD precautions or checking software before physical connections). Fix it yourself.

ANS:

- Power off, unplug the PC, and use an anti-static wrist strap or discharge static electricity.

- Check power cable, PSU switch, and motherboard/CPU power connectors.

- Check motherboard connections and POST indicators.

- Reseat RAM and verify it is detected in BIOS.

- Check SSD/HDD connections and BIOS detection.

- Power on and confirm successful POST.