- A user reports their laptop won't power on even when plugged in. List 5 troubleshooting steps you would take to diagnose the cause, and for each step, explain what you are checking for.

ANS:

| Step | Troubleshooting Action | What I am Checking |
| --- | --- | --- |
| 1. Check the power adapter | Connect the charger and check the charging indicator LED. | Checks whether the charger is supplying power and whether the adapter/cable is damaged. |
| 2. Check the wall socket | Try a different working electrical outlet. | Confirms that the problem is not caused by the power source. |
| 3. Perform a power reset | Disconnect the charger, remove the battery if removable, hold the power button for about 15–20 seconds, then reconnect the charger and try again. | Clears residual electrical charge and can recover a system that is stuck in a power state. |
| 4. Check the battery | Try starting the laptop with the charger connected; if possible, test with a known-good compatible battery/adapter. | Determines whether the battery is completely discharged or faulty. |
| 5. Check internal hardware | If there is still no response, a technician can inspect the DC-in jack, motherboard power circuit, and internal connections. | Looks for a damaged charging jack, loose connection, or motherboard/power-circuit failure. |

- You receive a helpdesk ticket: 'My monitor is showing a blank screen, but the CPU seems to be running.' Write a step-by-step troubleshooting flow you would follow to identify and resolve the issue.<br><br><em><strong>Hint:</strong> Consider checking both hardware connections and basic display settings.</em>

ANS:

- Check display cable

- Check the monitor input/source

- Check the correct graphics port

- Test with another monitor

- Check Windows display settings

- Check the graphics driver

- A user complains that their wireless mouse is unresponsive, but the keyboard works fine. Describe how you would isolate whether the issue is with the mouse, the USB port, or the system settings.

ANS:

- Check the mouse power

- Check the wireless receiver

- Test another USB port

- Test the mouse on another computer

- Check Windows settings and drivers

- Check for interference/pairing

- Imagine you are working on a helpdesk team for a gaming cafe. One system is overheating and shutting down during use. List 3 possible hardware causes and describe how you would check each one.<br><br><em><strong>Hint:</strong> Think about fans, dust, and thermal paste.</em>

ANS:

| Hardware Cause | How I Would Check It |
| --- | --- |
| 1. Faulty or slow CPU/GPU fan | Open the case and check whether the CPU and GPU fans spin properly when the system is running. Listen for unusual fan noise and check temperatures using monitoring software. |
| 2. Dust buildup | Inspect the CPU heatsink, GPU heatsink, case fans, and air vents for dust. Heavy dust can restrict airflow. Power off and unplug the PC before cleaning it with appropriate compressed air. |
| 3. Dried or improperly applied thermal paste | If temperatures remain high even with good airflow and working fans, inspect the CPU heatsink installation. Old or dried thermal paste may prevent proper heat transfer; a technician can clean and reapply suitable thermal paste. |